APPENDIX W

SOURCE CODE LISTING OF PERSONAL COMPUTER PROGRAM FOR 5 pH POINT TITRATION METHOD

# This appendix contains the listing of the source code for the source code file: 5ppt.pas. This file is available on the floppy disk whi·ch is attached to the inside of the thesis cover. The program is coded using Turbo Pascal Ver 4.0. It allows the calculation of H2C03*alkalinity, SCFA (as AT) and systematic pH measurement error from the data collected in the 5 pH point titration procedure. (For 5 pH point titration procedure, see Appendix V).

program atct;

(5 pH POINT TITRATION PROGRAM}

{This program calculates H2C03-alkalinity and short-chain fatty acid concentrations in aqueous solutions containing the carbonate, SCFA, ammoni'-'ll and phosphate subsystems. The total species concentrations of the ammoni'-'ll and phosphate subsystems need to be entered into the c ter program. The sample is titrated from its initial pH to four further pH values measured in the pH region: 6.7, 5.9, 5.2, 4.3. The program provides an estimate of a possible systematic pH measurement error in the event that the carbonate system dominates

over the remaining weak acid/base subsystems.}

{PROGRAM START:}

uses crt;

type phstr= stringt50J;

var pHO, pH1, pH2, pH3, pH4, delph, pHcorr, A1, B1, A2, B2,

HCt1, HCt2, Ct2, Ct1, Ctcomp, HAt1, HAt2, At1, At2, CtAtratio, dil, vx1, vx2, vx3, vx4, vxfi, vxs, ca, temp, ktemp, TDS, logf1, logf2, pK1, pK11, pK2, pK22, pKn, pKnn, Nt,

pKp, pKpp, Pt, pKa, pKaa, At,

HH2C03alksam, H2C03alksam, Hcarb01, Mcarb02, counter, ionstr, nuiber, vsdil, vsundil :real;

again, move,intro, x, y, a, b, c, d: char; labels: array[1..181 of phstr;

value: array[1..18l of real; 1,11,posy: Integer;

function log(x:real):real; begin log:=ln(x)/2.302585093; end;

function tento(y:real):real; begin

tento:=exp(y1'ln(10)); end;

function nue (TDS,dil:real):real; begin

Ill.le:= 0.000025* (TOS/dil-20); end;

function logf (mue,ktemp:real):real; begin

logf:= - 1.825 • 1000000 • exp(·1.5*ln(78.3*ktemp)) • (sqrt(111Je)/(1+­ sqrt(111Je)) • 0.3*111.le);

end;

function perH2C03 (ph:real):real; begin

perH2CO3:= 1/(1 + tento(·pk11)/tento(·ph) + tento(·pk11) * tento (·pk22)/tento(-2*ph));

end;

function perHCO3 (ph:real):real;

begin

perHCO3:= 1/(tento(·ph)/tento(·pk11) + 1 + tento(·pk22)/tento (·ph));

end;

function perCO3 (ph:real):real; begin

perC03:=1/(tento(·2*ph)/(tento(·pk11)*tento(·pk22)) + tento C·ph)/tentoC·pk22) + 1);

end;

function per(ph,pkk:real):real; begin

per:=tento(·pkk)/(tento(·pkk)+tento(·ph)); end;

function dH2CO3alk(pHf,pHs: real): real; begin

dH2CO3alk:= perHC03(pHf)·perHCO3(pHs) + 2 * (perCO3(pHf)·perC03(pHs)); end;

function dHAcalk(pHf,pHs: real): real; begin

dHAcalk:= per(pHf,pkaa) • per(pHs,pkaa); end;

function MH20 (vxfi,vxs,pHfi,pHs: real):real; begin

MH2O:= (vsdil+vxs)*tento(·pHs)/tento(logf1) • (vsdil+vxfi)*tento(·pHfi)/ tento(logf1) + Cvsdil+vxfi)*tento(CpHfi-14)) • (vsdil+vxs)* tento((p s-14));

end;

function MNH3(pHf,pHs:real):real; begin

MNH3:= Nt/(14OOO*dil) * vsdil * (per(pHf,pKnn) • per(pHs,pKnn));

end;

function MHP04CpHf,pHs:real):real; begin

MHP04:= Pt/(31OOO*dil) • vsdil * (per(pHf,pKpp) • per(pHs,pKpp)) end;

procedure mark; begin

textbackground(white); textcolor(black);

end;

procedure urmark;

begin textbackground(black); textcolor(whi te);

end;

procedure box(x1,y1,x2,y2,a:integer); var

!!:integer; begin

a:= x2-x1·1;

FOR 11:=1 to a DO	{draw the horizontal lines}

begin GOTOXY(x1+1,y1);

writeln(chr(196)); GOTOXY(x1+1,y2);

writeln(chr(196));

x1:=x1+1;

end;

x1:=x1·a; a:=y2-y1·1;

FOR 11:=1 to a DO

begin GOTOXY(x1,y1+1);

writeln(chr(179)); GOTOXY(x2,y1+1);

writeln(chrC179)); y1:=y1+1;

end; y1:=y1·a;

GOTOXY(x1,y1)j

writeln(chrC218)); GOTOXY(x1,y2);

writeln(chr(192)); GOTOXY(x2,y1)j

writeln(chr(191)); GOTOXY(x2,y2);

writeln(chr(217)); end; {procedure box}

procedure introscreen; begin

clrscr; mark;

{draw the vertical lines}

{draw the corners}

box(10,3,70,20,1); urmark; box(24,6,56,8,1);box(22,5,58,9,1); box(26,10,55,16,1);	-

GOTOXY(25,7); highvideo;

write(1FIVE pH POINT TITRATION METHOO');normvideo; GOTOXY(28,11);

write(1FOR DETERMINATION OF:1); GOTOXY(28,13);

write('(1) SHORT-CHAIN FATTY ACIDS'); GOTOXY(28,15);

write('C2) H2C03*ALKALINITY1); GOTOXY(11,16);

GOTOXY(12,18);

write('Copyright: UCT'); box(10,22,41,24,1); GOTOXY(12,23);

write('Press any letter to continue'); intro:= readkey;

end; {procedure introscreen}

procedure initlabel; begin

labelsC1J:='pHo (initial pH)•••••••••••••••••; labelsC2J:='pH1 (after adding Vx1)	•;

labels[3J:='pH2 (after adding Vx2)	•;

labels[4]:='pH3 (after adding Vx3)	•;

labelsC5J:=1pH4 (after adding Vx4)	•;

labels[6J:=1	Vx1 (ml)	•;

labelsCn:=1	Vx2 (ml)	•;

labelsC8J:='	Vx3 (ml)	•;

labels[9J:=•	Vx4 (ml)	•;

labelsC10J:='Normality of titrant (mol/l)	•;

labelsC11J:=1Sample size: undiluted (ml)	•;

labelsC12J :='Sample size: diluted (ml)	•;

labelsC13J :='Terrperature (Celsius)	•;

labelsC14J:='TDS (mg/l)	•;

labels[15J:=1Specific Conductivity (mS/m)	•;

labels[16J:='lnorganic Nitrogen (mgN/l)	•;

"· 5

labelsc1n:=1lnorganic Phosphorus (mgP/l)	';

end; {procedure initlabel}

procedure default_values; begin

value[1]:= 7.36;

value[2]:= 6.75;

value[31:= 5.95;

value[4]:= 5.18;

value[5J:= 4.29;

value[6]:= 1.06;

value[7]:= 3.50;

value[8]:= 4.84;

value[91:= 5.40;

value[10]:= 0.0728;

value[111 := 10;

value[12]:= 50;

value[13]:= 21;

value[14]:= 3300;

value[15J:= 488; value[16]:= O; valueC1n:= O;

end; {procedure default_values}

procedure allocate_data;

begin

pHO:= value[1l; pH1:= value[2]; pH2:= value[3]; pH3:= value[4]; pH4:= value[5J; vx1:= value[6]; vx2:= valueen; vx3:= value[8]; vx4:= value[9J; ca:= value[10J; vsundil:= value[11J; vsdil:= value[12]; tefl1):= value[13]; TOS:= value[14J; ionstr:= value[15]; Nt:= value[16J;

Pt:= valueC1n;

end; {allocate_data}

procedure write_value; begin GOTOXYC37,2+posy);

case posy of

1,2,3,4,5,6,7,8,9: begin

writeln(value[posy]:1:2);

end;

10: begin

writeln(value[posy]:1:4); end;

11,12,13,14,15,16,17,18: begin

writeln(value[posy]:1:0);

end; else end;

end; {procedure write_value}

procedure screen; begin

mark; box(50,2,79,8,1); urmark;

box(50,10,79,19,1);

boxC50,10,59,19,1);

box(50,10,79,13,1); GOTOXY(52,4);

11.6

write(' 5 pH POINT TITRATION '); GOTOXYC52,6);

write(' TITRATION INPUT DATA'); GOTOXY(52,12);

writeln(' KEY	FUNCTION'); GOTOXY(52,14);

writeln('	1,CHR(24),CHR(25),' SELECT PARAMETER');

GOTOXYC52,15);

writeln('<enter> ERASE VALUE'); GOTOXY(52,16);

writeln('<enter> INSERT NEIi VALUE'); GOTOXY(52,17);

writeln(' C	CALCULATION'); GOTOXY(52,18);

writeln(' Q	QUIT'); end; {procedure sreeen)

procedure restore;

begin clrscr; screen;

mark; box(1,1,47,22,1); unmark;

FOR I:= 1 to 17 Do

begin GOTOXY(3,2+I);

writeln(labels[ll ); GOTOXY(37,2+1);

case I of

1,2,3,4,5,6,7,8,9: begin

writeln(va Lue[ll:1:2);

end;

1D: begin

writeln(value[l]:1:4);

end;

11,12,13,14,15,16,17,18: begin

writeln(value [I]:1:0);

end;

else end;

end;  {FOR)

GOTOXY(3,2+posy);

mark; write(labels[posy]); write value;

unmark; end;

procedure display;

begin clrscr; screen;

mark; box(1,1,47,21,1); unnark;

FOR I:= 1 to 17 Do

begin

if 1=1 then mark;

GOTOXY(3,2+1);

writeln(labels[ll); GOTOXY(37,2+1);

case I of

1,2,3,4,5,6,7,8,9: begin

writeln(value[I]:1:2);

end;

10: begin

writeln(value(Il:1:4);

end;

11,12,13,14,15,16,17,18: begin

writeln(value[Il:1:0);

end;

else

end;

if 1=1 then unmark;

end; posy:=1; REPEAT

repeat move:= readkey until (upcase(move) in [1C1,1Q1]) or (ORD(move)

in [72,80,13]);

case 0RD(move) of 80: begin

G0T0XY(3,2+posy);

writeln(labels[posyl); write value; posy:;posy+1;

if posy=18 then posy:=1; G0T0XY(3,2+posy);

mark; writeln(labels[posy]); write_value; unmark; end;

72: begin

G0T0XY(3,2+posy);

writeln(labels[posyJ); write value; posy:;posy-1;

if posy=O then posy:=17; G0T0XY(3,2+posy);

mark; writeln(labels[posyJ); write_value; unmark; end;

13: begin

repeat G0T0XY(37,2+posy);

mark;

writeln('	'); GOT0XY(37,2+posy);

($!·} read(nuit>er) ($1+} until ioresult = O; value[posy]:= nunber; G0T0XY(37,2+posy);

unmark;

writeln('	'); write value;

case posy of 14: begin

value[posy+1J:= O.1488*(value[posyJ-20);

if value[posy+1J < 0 then value[posy+1J:= 0; GOTOXY(37,posy+3);

write('	'); G0T0XY(37,posy+3);

write(value[posy+1]:1:0); end;

15: begin

value[posy-1]:= 6.n•value[posyJ+20; G0T0XY(37,posy+1);

write('	'); GOT0XY(37,posy+1);

write(value[posy-1]:1:0); end;

else

end;

else end; end;

until upcase(move) in c•c1,1Q1];

clrscr; writeln('Calculating, please wait');

end; (procedure display}

procedure p(; begin

kt := 273 + t ;

if TDS < 20 then TDS:= 21; logf1:= logf(111Je(TDS,dil),kt ); logf2:= 4*logf1;

pK1:= ·1*(-356.3094 - 0.06091964*kt + 21834.37/kt + 126.8339

*log(kt ) - 1684915/(kt *kt )); pK11:= pK1 + logf1;

pK2:= ·1*(·107.8871 - 0.03252849*kt + 5151.79/kt + 38.92561

*log(kt ) - 563713.9/(kt *kt >>; pK22:= pK2 - logf1 + logf2;

pKa:= 1170.5/kt - 3.165 + 0.0134 * kt ;

pKaa:= pKa + logf1;	.

pKn:= 2835.8/kt -0.6322 + 0.00123 * kt ;

pKnn:= pKn + logf1 ;

pKp:= 1979.5/kt - 5.3541 + 0.01984 * kt ;

pKpp:= pKp - logf1 + logf2 end; {procedure pK}

procedure deltapH; begin

pHO:= pHO + pHcorr; pH3:= pH3 + pHcorr; end;

pH1:= pH1 + pHcorr; pH4:= pH4 + pHcorr;

pH2:= pH2 + pHcorr;

procedure atctcalculation; begin

A1:= (vx2-vx1)*ca • MH20(vx1,vx2,pH1,pH2) - MNH3(pH1,pH2) - MHP04(pH1,pH2) + dHAcalk(pH1,pH2)/dHAcalk(pH3,pH4) * (MH20(vx3,vx4,pH3,pH4) + MNH3(pH3,pH4) + MHP04(pH3,pH4) -

(vx4-vx3)*ca);

B1:= dH2C03alk(pH1,pH2) • dHAcalk(pH1,pH2)/dHAcalk(pH3,pH4) * dH2C03alk(pH3,pH4);

A2:= (vx4-vx1)*ca - MH20(vx1,vx4,pH1,pH4) - MNH3(pH1,pH4) -

MHP04(pH1,pH4) + dHAcalk(pH1,pH4)/dHAcalk(pH3,pH4) * (MH20(vx3,vx4,pH3,pH4) + MNH3(pH3,pH4) + MHP04(pH3,pH4) •

(vx4-vx3)*ca);

B2:= dH2C03alk(pH1,pH4) - dHAcalk(pH1,pH4)/dHAcalk(pH3,pH4) * dH2C03alk(pH3,pH4);

Ct1:= (A1/B1)/vsdil * 50000 * dil; Ct2:= (A2/B2)/vsdil * 50000 * dil; MCt1:= A1/B1;

Ctc°"":= Ct1 • Ct2;

end; {procedure atctcalculation}

procedure atct1; begin

dil:= vsdil/vsundil;

{us,ing pH: 1-2; 3-4}

{using pH: 1-4; 3-4}

pK; delpH:= O; pHcorr:=0; atctcalculation; counter:= O;

MAt1:= 1/dHAcalk(pH3,pH4) * ((vx4·vx3)*ca·MCt1*dH2C03alk(pH3,pH4) • MNH3(pH3,pH4) - MHP04(pH3,pH4) • MH20(vx3,vx4,pH3,pH4));

At1:= MAt1/vsdil * 60000 * dil; CtAtratio:= At1/Ct1;

if etc°""= 0 then x:= 1d1; if Ctc°"" > 0 then x:= •a•; if etc°""< 0 then x:= 1b1;

if CtAtratio > 0.5 then x:= •c•;

case x of

•a•: begin

pHcorr:= -0.01; repeat

delpH:= delpH • 0.01;

deltapH; counter:= counter+ 1; atctcalculation;

until (Ctc°"" < 0) or (counter> 19);

end;

1b1:  begin

pHcorr:= 0.01; repeat

else

end;

delpH:= delpH + 0.01;

deltapH; counter:= counter+ 1; atctcalculation;

until (Ctcomp > 0) or (counter> 19);

end;

MAt1:= 1/dHAcalk(pH3,pH4) * ((vx4-vx3)*ca-MCt1*dH2C03alk(pH3,pH4) - MNH3(pH3,pH4) • MHP04(pH3,pH4) - MH20(vx3,vx4,pH3,pH4));

At1:= MAt1/vsdil * 60000 * dil;

MAt2:= 1/dHAcalk(pH1,pH4) * ((vx4-vx1)*ca·MCt1*dH2C03alk(pH1,pH4) - MNH3(pH1,pH4) • MHP04(pH1,pH4) • MH20(vx1,vx4,pH1,pH4));

At2:= MAt2/vsdil * 60000 * dil;

MH2C03alksam:= MCt1 * (perHC03(pH0) + 2 * perC03CpH0));

H2C03alksam:= MH2C03alksam / vsdil * dil * 50000 + (tento(CpH0-14)) - tento(-pH0)/tento(logf1)) * 50000;

end; {procedure atct1}

procedure output; begin

clrscr;

mark; box(10,2,60,4,1);unmark; box(2,7,73,10,1); box(49,7,73,10,1);

box(2,11,73,14,1); box(49,11,73,14,1);

box(2,15,73,18,1); box(49,15,73,18,1); G0T0XY(26,3);

highvideo; write('OUTPUT DATA'); normvideo; G0T0XY(5,8);

write('H2C03*alkalinity (undiluted saq,le)'); G0TOXY(58,8);

highvideo; write(H2C03alksam:3:0); normvideo;

G0T0XY(5,9);	1

write('(mg/l as CaC03) ); G0T0XY(5,12);

write('Short-chain fatty acids (undiluted saq,le)'); G0TOXY(58,12);

if At1 > 0 then begin

highvideo; write(At1:3:0); normvideo; end

else begin

highvideo; write('0 ');normvideo; end;

G0TOXY(5,13);

write('(mg/l as acetic acid)'); G0TOXY(5,17);

write(1Systematic pH error•); G0TOXY(58,17);

highvideo; write(delpH:1:2); normvideo; if counter= 20 then

begin

G0TOXYC5,16);

write('The titration data indicate a systematic'); G0TOXYC5,17);

writeC'pH error> 0.2; Check pH probe calibration'); G0TOXYC58,17);

write('	1 );

end;

if x = •c• then begin G0TOXYC5,16);

write('Correction for systematic pH error'); G0TOXY(5,17);

write('is not possible for this titration'); G0T0XY(60,17); •

write('	'); end;

end; {procedure output}

procedure exitbox;

begin

repeat;

box(S,20,60,24,1); box(49,20,60,22,1);

box(S,22,60,24,1); box(49,22,60,24,1); GOTOXY(7,21 );

write('Do you wish to do a further calculation 71); GOTOXY(53,21);

highvideo; write('Y'); normvideo; write('/'); highvideo; write ('N');

normvideo;

GOTOXY(7,23);

write('Do you wish to quit the program?'); GOTOXY(54,23);

highvideo; write('Q'); normvideo; again:=readkey; again:=upcase(again); until again in [1Y1,1N1,1Q1J;

clrscr;

end; {procedure exitbox}

begin	C of main program} introscreen;

again:= 'Y'; default_values;

repeat; clrscr; initlabel; display; if upcase(move) in ['C'l then begin

allocate_data; atct1; output; exitbox; end

else begin

clrscr; again:= 'N';

-end;

until agein in c•N 1 , 1 Q•J ;

end. {of main program}

{PROGRAM END.}